# Question- Explain the operational differences between static and dynamic routing, and justify why configuring static routes would be the most efficient choice in this scenario.

Static Routing vs Dynamic Routing

| Feature | Static Routing | Dynamic Routing |
| --- | --- | --- |
| Configuration | Manually configured by administrator | Learned automatically using routing protocols |
| Route changes | Administrator must update routes | Routes automatically adapt to changes |
| Protocols | No routing protocol required | RIP, OSPF, EIGRP, BGP, etc. |
| Resource usage | Very low CPU, RAM, and bandwidth usage | Uses CPU, RAM, and network bandwidth |
| Complexity | Simple for small networks | More complex to configure/manage |
| Failure recovery | Usually requires manual intervention | Can automatically find an alternative path |
| Best suited for | Small, stable networks | Large or frequently changing networks |

## Why Static Routing is better for this startup

The startup has only two routers connecting two offices, and the topology rarely changes. Therefore, static routing is the more efficient choice.

For example:

Office A	Office B

PCs ── Switch ── R1 ───────── R2 ── Switch ── PCs Link

You could manually configure a route on R1 pointing to Office B's network through R2, and a route on R2 pointing back to Office A's network through R1.

For example:

R1 → Office B network → R2 R2 → Office A network → R1

## Why this makes sense

- Very small network — Only two routers means there are very few routes to configure.

- Stable topology — Since the network rarely changes, automatic route discovery isn't necessary.

- Less overhead — No RIP/OSPF/EIGRP routing updates are needed.

- Simple management — The administrator can easily understand and troubleshoot the routes.

- Predictable routing — Traffic follows the manually defined path.

- No protocol configuration — You don't need to configure a dynamic routing protocol.

Static routing is manually configured, while dynamic routing uses routing protocols to automatically learn and update routes. In this scenario, static routing is more efficient because there are only two routers and the topology rarely changes. Static routes require minimal resources, are simple to configure, and avoid the overhead of running a dynamic routing protocol. Dynamic routing would become more useful if the network grew or routes changed frequently.

# Question: Compare RIP and OSPF as dynamic routing protocol options for this network, and recommend which one is better suited as the network grows, justifying your choice based on convergence time and scalability.

For a network with five branch routers, both RIP and OSPF can provide dynamic routing, but their behavior and scalability are different.

| Feature | RIP | OSPF |
| --- | --- | --- |
| Routing type | Distance-vector | Link-state |
| Metric | Hop count | Cost (based mainly on bandwidth) |
| Maximum hops | 15 hops | No 15-hop limitation |
| Convergence | Relatively slow | Fast |
| Scalability | Suitable for small networks | Suitable for medium/large networks |
| Updates | Periodic routing updates | Sends updates mainly when topology changes |
| Network efficiency | More overhead as network grows | More efficient for larger networks |
| Complexity | Easier to configure | More complex |

## Convergence time

Convergence means how quickly all routers learn about a network change or failure. Example:

### R2

/ \

### R1	R3

\	/

### R4

Suppose the link between R1 and R2 fails.

- RIP: Routers exchange routing information periodically, so it can take longer to recognize and adapt to the failure.

- OSPF: Routers detect the topology change and distribute link-state information, allowing the routing tables to be recalculated much faster.

## So, OSPF generally converges faster than RIP. Scalability

With five routers, RIP can still work:

R1 ── R2 ── R3

│

R4 ── R5

But as the start up adds more branches, RIP becomes less suitable because of its 15-hop limit and less efficient routing behavior.

OSPF is designed to scale much better. It can organize larger networks using areas, reducing the amount of routing information that each router needs to process.

## Recommendation: OSPF

For this start up, I would choose OSPF as the network grows.

## Main reasons:

- ) Faster convergence — It responds more quickly to link/network failures.

- ⬛#/ Better scalability — It can handle growth much better than RIP.

- ~—Eç"-¯˙·;− < Better path selection — OSPF uses cost rather than simply counting hops.

- ⬛³– Efficient updates — It doesn't rely on constantly sending the complete routing table like traditional RIP operation.

- c.…   Suitable for future expansion — OSPF's area structure makes it appropriate as the number of branches increases.

RIP is simpler and can work well for small networks, but it has a 15-hop limitation and slower convergence. OSPF uses a link-state approach, provides faster convergence, and scales much better as the network grows. Since the start up is expanding from two routers to five and may continue growing, OSPF is the better choice because it provides faster recovery from topology changes and better scalability.

# Question: Explain how the switch populates its MAC address table and forwards frames between the two devices, including what happens before the table is populated.

When two devices are connected to the same switch and communicate for the first time, the switch uses MAC addresses to learn where each device is located.

## Example topology

### PC1	PC2

MAC: AA-AA-AA		MAC: BB-BB-BB IP: 192.168.1.10	IP: 192.168.1.20

│	│

└─────── Switch ───────┘

│

MAC Address Table

## Initially, the MAC table is empty

When the switch starts, it doesn't know which MAC address belongs to which port. MAC Address Table

MAC Address	Port

(empty)

## PC1 sends a frame

Suppose PC1 wants to communicate with PC2. PC1 sends an Ethernet frame:

Source MAC	= AA-AA-AA

Destination MAC = BB-BB-BB

The frame enters the switch through Port 1.

## Switch learns PC1's MAC

The switch looks at the source MAC address. It learns:

"AA-AA-AA came from Port 1." So it adds:

MAC Address	Port

AA-AA-AA	Fa0/1

This process is called MAC address learning.

## But the destination MAC is unknown

The switch checks:

Destination = BB-BB-BB

But BB-BB-BB isn't in its MAC table yet.

So the switch doesn't know which port PC2 is connected to. It performs unknown unicast flooding.

┌── PC2

│

PC1 ── SW ───┼── Other ports

│

└── Other ports

The switch sends the frame out of the other appropriate ports except the port where the frame arrived.

PC2 receives the frame and processes it if the destination MAC matches.

## PC2 replies

PC2 sends a reply back to PC1:

Source MAC	= BB-BB-BB

Destination MAC = AA-AA-AA

The switch receives this on PC2's port and learns:

MAC Address	Port

AA-AA-AA	Fa0/1

BB-BB-BB	Fa0/2

Now the switch knows where both devices are.

## Future communication is unicast

If PC1 sends another frame to PC2:

### PC1

│

│ Destination = BB-BB-BB

▼

Switch

│

└──────────────► Fa0/2 → PC2

The switch looks up BB-BB-BB in its MAC table and sends the frame only through Fa0/2. It does not flood the frame to every port.

A switch learns MAC addresses by examining the source MAC address of incoming Ethernet frames and associating that MAC address with the incoming port. When a destination MAC is not yet in the MAC address table, the switch floods the frame out of all ports except the incoming port. Once the destination device responds, the switch learns its MAC address as well. After both MAC addresses are learned, the switch forwards subsequent frames directly to the correct port instead of flooding them.

# Question: Explain the purpose of VLAN segmentation, and describe how an 802.1Q trunk link allows traffic for multiple VLANs to travel between the switch and the router over a single physical connection.

In this startup, Marketing and Engineering use the same physical switch, but we want their traffic to be logically separated.

## What is VLAN segmentation?

A VLAN (Virtual LAN) divides one physical switch into multiple logical networks. Example:

### SWITCH

┌─────────┴─────────┐

│	│

### VLAN 10	VLAN 20

Marketing	Engineering

192.168.10.0	192.168.20.0

│	│

### PC1, PC2	PC3, PC4

Even though all PCs are connected to the same physical switch, they belong to different logical networks.

## Why use VLANs?

- Security / Isolation

Marketing devices don't automatically belong to the same Layer-2 network as Engineering devices.

## Broadcast control

A broadcast from Marketing stays within VLAN 10 instead of reaching Engineering's VLAN 20.

## Organization

You can easily assign:

  - VLAN 10 → Marketing

  - VLAN 20 → Engineering

## Better network management

Each department can have its own IP subnet and security policies.

- What is an 802.1Q trunk?

Now suppose Marketing and Engineering need to communicate with other networks through one router.

Instead of using two separate physical cables:

Switch ───── Router ← Marketing Switch ───── Router  ← Engineering

we can use one physical connection: 802.1Q TRUNK

Switch ===================== Router VLAN 10 + VLAN 20

The trunk carries traffic belonging to multiple VLANs over the same physical link.

- How does the router know which VLAN traffic belongs to?

This is where 802.1Q tagging is used.

When a frame travels across the trunk, an 802.1Q VLAN tag identifies its VLAN. For example:

Marketing frame

↓

[ Ethernet Frame | VLAN 10 Tag ]

↓

### TRUNK

↓

Router And:

Engineering frame

↓

[ Ethernet Frame | VLAN 20 Tag ]

↓

### TRUNK

↓

Router

So the same cable can carry:

VLAN 10 ─┐

├── One physical trunk ── Router VLAN 20 ─┘

- Router-on-a-Stick

A common setup for this scenario is called Router-on-a-Stick. Marketing PCs

### VLAN 10

│

│

▼

┌────────┐

│ SWITCH │

└───┬────┘

│

### │ 802.1Q TRUNK

│ VLAN 10 + VLAN 20

│

┌───▼────┐

│ ROUTER │

│	│

│ VLAN10 │ → Marketing gateway

│ VLAN20 │ → Engineering gateway

└────────┘

▲

│

Engineering PCs VLAN 20

The router uses subinterfaces such as:

G0/0.10 → VLAN 10 G0/0.20 → VLAN 20

This allows the router to perform inter-VLAN routing.

VLAN segmentation logically separates departments on the same physical switch. For example, Marketing can use VLAN 10 and Engineering can use VLAN 20. This provides logical isolation and reduces unnecessary broadcast traffic. An 802.1Q trunk allows multiple VLANs to travel over one physical link between the switch and router by adding VLAN tags to Ethernet frames. The router can use separate subinterfaces for each VLAN and perform inter-VLAN routing.

# Question: Explain why inter-VLAN routing is required here, and justify the specific advantages of a Router-on-a-Stick configuration over adding a second physical router interface.

## Inter-VLAN Routing

In this scenario, the startup has two VLANs:

- Marketing VLAN → e.g. VLAN 10

- Engineering VLAN → e.g. VLAN 20

- Both VLANs are connected to the same switch

- There is only one router

Why is Inter-VLAN Routing required?

By default, different VLANs are separate Layer-2 broadcast domains. For example:

Marketing VLAN 10		Engineering VLAN 20 PC1 ────────┐	┌──────── PC3

│	│

[ Switch ]

│

VLAN 10 ≠ VLAN 20

A normal Layer-2 switch can forward frames within the same VLAN, but it cannot route traffic from VLAN 10 to VLAN 20.

So if:

### PC1 (VLAN 10)

IP: 192.168.10.10

↓

### PC3 (VLAN 20)

IP: 192.168.20.10

PC1 needs a Layer-3 device to move the packet between the two different networks. That is the job of inter-VLAN routing.

Why Router-on-a-Stick?

Instead of adding another physical router interface, we can use one physical router interface divided into multiple logical subinterfaces.

Example:

Router G0/0

│

│ Trunk

│

[ Switch ]

/	\

### VLAN 10	VLAN 20

Marketing	Engineering On the router:

G0/0.10 → VLAN 10 → 192.168.10.1

G0/0.20 → VLAN 20 → 192.168.20.1

The switch sends VLAN-tagged traffic through the single trunk link, and the router's subinterfaces route traffic between the VLANs.

## Main advantages

- Saves hardware

You don't need a separate physical router interface for every VLAN. Traditional:

VLAN 10 → Physical Interface

VLAN 20 → Physical Interface VLAN 30 → Physical Interface

With Router-on-a-Stick:

VLAN 10 ─┐

VLAN 20 ─┼──→ ONE physical router interface VLAN 30 ─┘

## Lower cost

For a small startup, using one existing router avoids buying additional router hardware or interface modules.

## Easy to expand

If the company adds another VLAN, you can create another subinterface rather than installing another physical interface.

Example:

G0/0.10 → Marketing G0/0.20 → Engineering G0/0.30 → HR

G0/0.40 → Finance

## Centralized configuration

All inter-VLAN routing can be configured on the same router.

## Router-on-a-Stick vs Multiple Physical Interfaces

| Router-on-a-Stick | Multiple physical interfaces |
| --- | --- |
| One physical interface | One physical interface per VLAN |
| Uses subinterfaces | Uses physical interfaces |
| Lower hardware cost | More hardware required |
| Easy to add VLANs | Limited by available interfaces |
| Requires trunk link | Separate links/interfaces |
| Suitable for small networks | Can provide better performance at higher traffic |

## One important disadvantage

Router-on-a-Stick has one physical link carrying traffic for multiple VLANs. Therefore, that link can become a bottleneck if the network has heavy inter-VLAN traffic.

For this small startup with a single router, however, Router-on-a-Stick provides a practical way to enable Marketing ↔ Engineering communication without adding another physical router interface.

“Inter-VLAN routing is required because VLANs are separate Layer-2 broadcast domains and a Layer-2 switch cannot route traffic between them. Router-on-a-Stick allows one physical router interface to provide routing for multiple VLANs using logical subinterfaces over a trunk link, reducing hardware cost and making the network easier to scale.”

# Question: Explain the wireless security and MAC filtering options available, and justify which configuration approach best prevents unauthorized access while still allowing convenient onboarding of new employee devices.

Wireless Security & MAC Filtering — Easy Explanation

In this scenario, the startup has an open-plan office where unauthorized devices have connected before

## Only employees should be able to access the Wi-Fi, while adding a new employee device should remain easy.

- Wireless Security Options

| Security | How it works | Security level |
| --- | --- | --- |
| Open | No password | ❌ Very poor |
| WEP | Old encryption method | ❌ Insecure/obsolete |
| WPA/WPA2- Personal | Shared Wi-Fi password | ✅ Good |
| WPA3-Personal | Modern password-based security | ✅✅ Strong |
| WPA2/WPA3- Enterprise | Individual employee authentication through authentication server | ✅✅✅ Best for organizations |

For a startup, WPA3-Personal is a strong choice if all employee devices support it. If compatibility is an issue, WPA2/WPA3 mixed mode can be considered.

Every network device has a MAC address. Example:

Employee Laptop

### MAC: AA:BB:CC:11:22:33

The access point can maintain a list of allowed MAC addresses.

## Allow-list

Allowed:

AA:BB:CC:11:22:33 → Laptop 1 AA:BB:CC:44:55:66 → Laptop 2 AA:BB:CC:77:88:99 → Phone

Only devices on the list are allowed to connect.

## Deny-list

You can also specifically block known devices. Blocked:

### XX:XX:XX:XX:XX:XX

Best Configuration for This Scenario

For a startup that wants security + convenient employee onboarding, I would use:

WPA2/WPA3-Enterprise + individual employee authentication

Ideally:

Employee Device

↓

Wi-Fi AP

↓

WPA2/WPA3-Enterprise

↓

Authentication Server

↓

Employee credentials

↓

Access

Each employee can have their own credentials, rather than everyone sharing one Wi-Fi password.

Why this is convenient When a new employee joins: New Employee

↓

Create employee account

↓

Configure Wi-Fi on laptop/phone

↓

Authenticate

↓

Access granted

When an employee leaves: Employee leaves

↓

Disable their account

↓

Their Wi-Fi access stops

You don't need to change the Wi-Fi password for the entire company.

If the startup specifically wants MAC filtering, use it as an additional layer, preferably with an allow-list, rather than relying on it as the main security mechanism.

However, maintaining a MAC allow-list for every employee device can make onboarding and device replacement more inconvenient.

So the practical priority is:

Strong authentication first → MAC filtering as optional additional control.

“Open Wi-Fi and WEP should be avoided because they provide inadequate security. WPA2/WPA3 provides encrypted access, while WPA2/WPA3-Enterprise allows each employee to authenticate individually. MAC filtering can allow or block specific devices, but it should not be relied upon as the primary security mechanism because MAC addresses can be spoofed. For a startup, WPA2/WPA3-Enterprise with individual employee authentication provides strong security and convenient onboarding because new employees can be added without changing a shared Wi-Fi password.”