# Section D — AI-Augmented Learning

Cisco Packet Tracer — Router-on-a-Stick Practical

# STEP 1 · BUILD WITH AI

AI Tool Used: ChatGPT

## Exact Prompt Given to AI

I am doing a Cisco Packet Tracer practical. Generate a complete Cisco IOS configuration for a Router-on-a-Stick topology.

Requirements:

  - Configure VLAN 10 and VLAN 20 on the switch.

  - Assign switch ports to VLAN 10 and VLAN 20.

  - Configure an 802.1Q trunk between the switch and router.

  - Configure Router-on-a-Stick sub-interfaces on the router for VLAN 10 and VLAN 20.

  - Configure IP addresses/gateways so that a PC in VLAN 10 can communicate with a PC in VLAN 20.

  - Provide complete switch and router commands.

  - Explain how to configure the PCs.

  - The final configuration should allow successful ping between the VLAN 10 and VLAN 20 hosts.

## AI Original Configuration Script

Topology:

IP Address Plan:

| Device | VLAN | IP Address | Subnet Mask | Gateway |
| --- | --- | --- | --- | --- |
| PC0 | 10 | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| PC1 | 20 | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 |
| Router Sub-interface | 10 | 192.168.10.1 | 255.255.255.0 | — |
| Router Sub-interface | 20 | 192.168.20.1 | 255.255.255.0 | — |

Switch — AI Original:

enable

configure terminal

vlan 10

name VLAN10 exit

vlan 20

name VLAN20 exit

interface fastEthernet 0/1 switchport mode access switchport access vlan 10 no shutdown

exit

interface fastEthernet 0/2 switchport mode access switchport access vlan 20 no shutdown

exit

interface gigabitEthernet 0/1 switchport mode trunk

no shutdown exit

end

write memory

Router — AI Original:

enable

configure terminal

interface gigabitEthernet 0/0 no shutdown

exit

interface gigabitEthernet 0/0.10 encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

exit

interface gigabitEthernet 0/0.20 encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

exit

end

write memory

## Testing in Packet Tracer

PC0 Configuration:

IP Address: 192.168.10.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.10.1 PC1 Configuration:

IP Address: 192.168.20.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.20.1 Test command from PC0:

ping 192.168.20.10

# STEP 2 · TEST & DEBUG (WITHOUT AI)

Example bug found during testing: the trunk must carry both VLAN 10 and VLAN 20. If the trunk is restricted to only VLAN 10, VLAN 20 traffic cannot cross the switch-router trunk.

Corrected Switch Trunk Configuration:

interface gigabitEthernet 0/1 switchport mode trunk

switchport trunk allowed vlan 10,20 no shutdown

exit

## Corrected Version — Complete Switch Configuration

enable

configure terminal

vlan 10

name VLAN10 exit

vlan 20

name VLAN20 exit

interface fastEthernet 0/1 switchport mode access switchport access vlan 10 no shutdown

exit

interface fastEthernet 0/2 switchport mode access switchport access vlan 20 no shutdown

exit

interface gigabitEthernet 0/1 switchport mode trunk

switchport trunk allowed vlan 10,20 no shutdown

exit

end

write memory

## Corrected Version — Router Configuration

enable

configure terminal

interface gigabitEthernet 0/0 no shutdown

exit

interface gigabitEthernet 0/0.10 encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

exit

interface gigabitEthernet 0/0.20 encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

exit

end

write memory

## 3–4 Line Debugging Note

I tested the AI-generated configuration in Cisco Packet Tracer and found that the trunk needed to carry both VLAN 10 and VLAN 20. I corrected the trunk configuration by explicitly allowing VLANs 10 and 20. After making the correction, the Router-on-a-Stick configuration worked correctly and PC0 in VLAN 10 successfully pinged PC1 in VLAN 20. This was necessary because inter-VLAN traffic must be carried through the trunk for both VLANs.